These are the CAN logs from a 2015 Volkswagen Golf GTE (Mk.7, MQB) gathered from the TIUL coupling point (Coupling Points). Each file is a log of separate CAN connection

Timeline: Vehicle sleep => IGN on => engine on => engine off => IGN off.

## CAN connections

Connections tested, file timestamp from earliest:

T17e (upper pair)

Powertrain CAN - B384/B391

T17e (lower pair)

Running gear/dash panel insert CAN - B672/B676

T17d

Extended CAN - B108/B109

T17f

Infotainment / Hybrid CAN, not sure which one

The EPS (J500) is on the Running Gear CAN - connection B663 and B664, or Dash Panel Insert CAN - connection B672 and B676. (J500 Wiring Diagram)

## Connector PN

### T17g/T17o (E in diagram)

Female - 4F0 972 575 D
Male - 4F0 972 483 D

Pinout

15 - +12V 10amp from Fuse 10 in Block B straight to J533, cable 1mm diameter

### T17e/T17m (D in diagram)

Female - 4F0 972 575 A
Male - 4F0 972 483 A

#### Pinout

1 - CAN Drive		HI	B384

2 - CAN Drive		LO	B391

3 - CAN Chassis/IC	HI	B672

4 - CAN Chassis/IC	LO	B676

8 - Front Airbag Crash Sensor

9 - CAN ACC - HCA Direct

10 - Front Airbag Crash Sensor

11 - CAN ACC - HCA Direct

12 - +12V K15	a		A38

15 - EPB Motor Connection 3	B791

17 - EPB Motor Connection 4	B792

### T17d/T17l (G in diagram)

Female - 4F0 972 575
Male - 4F0 972 483

#### Pinout UNSURE

6 - CAN Convenience HI 	B398	/ GND? UNSURE

9 - +12V K15a			A20

11 - CAN Convenience LO	B407

12 - CAN Extended HI	B108

13 - CAN Extended LO	B109

### T17f/T17n (F in diagram)

Female - 4F0 972 575 B
Male - 4F0 972 483 B

#### EPS Connection

EPS connects to TML coupling point

Connector T6cj

Male - 4H0 973 713

Female - 3B0 973 813

Coupling point in engine compartment, left -TML- , only models with hybrid drive system

A - 14-pin connector -T14q- , black / 14-pin connector -T14t- , black

B - 14-pin connector -T14z- , grey / 14-pin connector -T14v- , grey

C - 8-pin connector -T8au- , black / 8-pin connector -T8az- , black

D - 14-pin connector -T14aa- , grey / 14-pin connector -T14ab- , grey

E - 14-pin connector -T14ac- , black / 14-pin connector -T14ad- , black